State Equations:

y= - ψx+2Cαmcosδδ-y+lfψx-y-lrψx

x=ψy+1mF-fg

ψ=2lfCαIzδ-y+lfψx-2lrCα Iz-y-lrψx

X=xcosψ-ysinψ

Y=xsinψ+ycosψ

Measurements:

y=xyψXYψ

Model Parameters:

δ≤π6 rad

F≥0 and F≤15367 N

x≥10-5 m/s

| Name | Description | Unit | Value |
| --- | --- | --- | --- |
| (x,y) | Vehicle’s velocity along the direction of vehicle frame | m/s | State |
| X,Y | Vehicle’s coordinates in the world frame | m | State |
| ψ,ψ | Body yaw angle, angular speed | rad,rad/s | State |
| δ, δf | Front wheel angle | rad | Input |
| F | Total input force | N | Input |
| m | Vehicle mass | kg | 1888.6 |
| lr | Length from rear tire to the center of mass | m | 1.39 |
| lf | Length from front tire to the center of mass | m | 1.55 |
| Cα | Cornering stiffness of each tire | N | 20000 |
| Iz | Yaw inertia | kg m2 | 25854 |
| Fpq | Tire Force, p ∈{x,y}, q ∈{f,r} | N | Depends on input force |
| f | Rolling resistance coefficient | N/A | 0.019 |
| delT | Simulation timestep | sec | 0.032 |

- Linearization

Lateral Linearization

u=δF, s1=yyψψ

Assume x, x are constant in the lateral system.

Let x=vx

Non-linearized System:

s1=yyψψ=y- ψvx+2Cαmcosδδ-y+lfψvx-y-lrψvxψ2lfCαIzδ-y+lfψvx-2lrCα Iz-y-lrψvx=f(s1,u)

A1= ∇fs1, B1=∇fu

A1=01000-2Cαmvxcosδ+10-vx+2Cαmvx-lfcosδ+lr001002CαIzvxlr-lf02CαIzvx-lf2-lr2

B1=002Cαm-sinδδ-y+lfψvx+cosδ0002lfCαIz0

Apply small angle Approximations →cosδ≈1, sinδ≈0

A1=01000-4Cαmvx0-vx-2Cαmvxlf-lr 001002CαIzvxlr-lf0-2CαIzvxlf2+lr2

B1=002Cαm0002lfCαIz0

Longitudinal Linearization

u=δF, s2=xx

Assume y, y,  ψ, ψ,  are constant in the longitudinal linearization

Non-linearized System:

s2=xx =xψy+1mF-fg

A2= ∇fs2, B1=∇f2

A2=0100

B2=0001m

Dynamics of rolling accommodate for using a disturbance term

dt=0ψy+fgm

PID Results:

Fig.1 Simulation Plot

Fig.2 Simulation Result

Twiddle Algorithm for PID tuning

import numpy as np

from util import getTrajectory, closestNode, clamp

default_dt = 0.032

class PID:

def __init__(self, Kp, Ki, Kd, dt=default_dt):

self.Kp = Kp

self.Ki = Ki

self.Kd = Kd

self.dt = dt

self.integral = 0

self.prev_error = 0

def reset(self):

self.integral = 0

self.prev_error = 0

def update(self, setpoint, measurement):

error = setpoint - measurement

self.integral += error * self.dt

derivative = (error - self.prev_error) / self.dt

output = self.Kp * error + self.Ki * self.integral + self.Kd * derivative

self.prev_error = error

return output

# Load trajectory using util.py

trajectory = getTrajectory('buggyTrace.csv')

# Simulate lateral controller (returns total squared cross-track error)

def lateral_error(pid_gains, trajectory, dt=default_dt):

Kp, Ki, Kd = pid_gains

pid = PID(Kp, Ki, Kd, dt)

X, Y = trajectory[0, 0], trajectory[0, 1]  # start at first waypoint

total_error = 0

for i in range(1, len(trajectory)):

cross_track_error, idx = closestNode(X, Y, trajectory)

control = pid.update(0, cross_track_error)  # setpoint is 0

# Simulate: move towards next waypoint, apply steering

direction = trajectory[i] - np.array([X, Y])

speed = 11 #

max_steer = np.radians(30)

control = clamp(control, -max_steer, max_steer)

X += direction[0] * dt + control * dt

Y += direction[1] * dt + control * dt

total_error += cross_track_error ** 2

return total_error

# Simulate longitudinal controller (returns total squared speed error)

def longitudinal_error(pid_gains, trajectory, desired_speed=11.0, dt=default_dt):

Kp, Ki, Kd = pid_gains

pid = PID(Kp, Ki, Kd, dt)

speed = 0.0

total_error = 0

for i in range(1, len(trajectory)):

speed_error = desired_speed - speed

control = pid.update(desired_speed, speed)

# Simulate: update speed

speed += control * dt

speed = clamp(speed, 0, 50)  # clamp to reasonable speed range

total_error += speed_error ** 2

return total_error

# Twiddle algorithm for PID tuning

def twiddle(error_func, trajectory, tol=0.01, initial_p=[0.00, 0.0, 0.0], initial_dp=[0.01, 0.01, 0.01]):

p = initial_p.copy()

dp = initial_dp.copy()

best_err = error_func(p, trajectory)

it = 0

prev_err = None

while sum(dp) > tol:

for i in range(len(p)):

p[i] += dp[i]

err = error_func(p, trajectory)

if err < best_err:

best_err = err

dp[i] *= 1.1

else:

p[i] -= 2 * dp[i]

err = error_func(p, trajectory)

if err < best_err:

best_err = err

dp[i] *= 1.05

else:

p[i] += dp[i]

dp[i] *= 0.95

it += 1

print(f"Iteration {it}, PID: {p}, Error: {best_err}")

# Stop if error does not change

if prev_err is not None and abs(best_err - prev_err) < 1e-8:

print("Error did not change, stopping Twiddle.")

break

prev_err = best_err

return p

if __name__ == "__main__":

print("Tuning lateral PID...")

best_pid_lateral = twiddle(lateral_error, trajectory)

print("Best lateral PID gains:", best_pid_lateral)

# dp's set to 0.1 - Best lateral PID gains: [0.1, 0.0, 0.0]

# dp s set to 0.01 - Best lateral PID gains: [0.11435888100000004, 0.0, 0.0178966525]

print("Tuning longitudinal PID...")

best_pid_longitudinal = twiddle(longitudinal_error, trajectory)

print("Best longitudinal PID gains:", best_pid_longitudinal)

# dp's set to 0.1- Best longitudinal PID gains: [29.91268053287073, 1.3640345468017792, 0.02870992997411423]

# dp's set to 0.01 -Best longitudinal PID gains: [27.580149049219923, 0.4822354299071474, 0.0734521331616762]

Tuned PID Controller:

# Import libraries

import numpy as np

from base_controller import BaseController

from scipy import signal, linalg

from util import *

class PID:

def __init__(self, Kp, Ki, Kd, dt):

self.Kp = Kp

self.Ki = Ki

self.Kd = Kd

self.dt = dt

self.integral = 0

self.prev_error = 0

def reset(self):

self.integral = 0

self.prev_error = 0

def update(self, setpoint, measurement):

error = setpoint - measurement

self.integral += error * self.dt

derivative = (error - self.prev_error) / self.dt

output = self.Kp * error + self.Ki * self.integral + self.Kd * derivative

self.prev_error = error

return output

# CustomController class (inherits from BaseController)

class CustomController(BaseController):

def __init__(self, trajectory):

super().__init__(trajectory)

# Define constants

self.lr = 1.39

self.lf = 1.55

self.Ca = 20000

self.Iz = 25854

self.m = 1888.6

self.g = 9.81

# Iter 1: 0.1, 0.0, 0.0

# Iter 2: 0.11435888100000004, 0.0, 0.0178966525

# Iter 3: reducing Kp a bit to reduce oscillations

self.lateral_pid = PID(Kp=0.1, Ki=0.0, Kd=0.0178966525, dt=0.032)

# Iter 1: 29.91, 1.36, 0.03

# Iter 2: 27.580149049219923, 0.4822354299071474, 0.0734521331616762

self.longitudinal_pid = PID(27.580149049219923, 0.4822354299071474, 0.0734521331616762, 0.032)

def update(self, timestep):

trajectory = self.trajectory

lr = self.lr

lf = self.lf

Ca = self.Ca

Iz = self.Iz

m = self.m

g = self.g

# Fetch the states from the BaseController method

delT, X, Y, xdot, ydot, psi, psidot = super().getStates(timestep)

# ---------------|Lateral Controller|-------------------------

# closest node

cross_track_error, idx = closestNode(X, Y, trajectory)

closest_node = trajectory[idx]

next_node = trajectory[(idx + 1) % len(trajectory)]  # safe wrap-around

# Path tangent vector

t_dx = next_node[0] - closest_node[0]

t_dy = next_node[1] - closest_node[1]

# Vector from path to car

v_dx = X - closest_node[0]

v_dy = Y - closest_node[1]

# Signed cross-track error

signed_cte = np.sign(t_dx * v_dy - t_dy * v_dx) * cross_track_error

# PID control

delta = self.lateral_pid.update(0, signed_cte)

delta = clamp(delta, -np.pi/6, np.pi/6)

# ---------------|Longitudinal Controller|-------------------------

# Many of the attempt changes were made by adjusting speed

desired_speed = 10.5

current_speed = np.sqrt(xdot**2 + ydot**2)

F = self.longitudinal_pid.update(desired_speed, current_speed)

F = clamp(F, 0, 15736)

# Return all states and calculated control inputs (F, delta)

return X, Y, xdot, ydot, psi, psidot, F, delta